B-ABC-09 (Aqua Based Cleaner)

EXPLANATION

B-ABC-09 elaborated to a specified degree to remove the no-clean and water-soluble flux residues from Pb-free and Lead/Tin and Polychlorinated biphenyls are organochlorine compounds with the formula C₁₂H₁₀₋ₓClₓ; they were once widely used in the manufacture of carbonless copy paper, as heat transfer fluids, and as dielectric and coolant fluids for electrical equipment

QUALITY AND ADVANTAGE

- Non tending to absorb moisture from the air and non-corrosive

- Tin, Copper, Gold, Cadmium, Zinc, Silver and Organic solderability preservative

- The Aquatic Based Cleaner "halogen-free" products are more environmentally friendly i,e Halogen and Halide – free

- Zero ion PCB and BCBA

- Highly Concentrated cleaner 1: 4.5 to 7 (Water) ratios can be mixed with DI or Reverse osmosis to get good results in manual, ultrasonic or conveyorized cleaning methods

- Tack-free, good cosmetics

- B-ABC-09 is perfect to use on the following metal surfaces, provided the surfaces are not heavily tarnished

PROPERTIES

Saleable/ Validity Period			540 Days

Ph	caps						8.5 +/- 0.4

Odour						Slightly Ammonia

Density  						0.995 g/cm3 at 25°C

Boiling point:					95°C

USAGE

This cleaner is easily dissolved in water. a preferably concentrated cleaner can be used in 1: 4 to 7 (Water) ratios for manual, ultrasonic, or conveyorized cleaning methods. Can go up to 25% of concentrated cleaner for strong residues, it is better to use Di ionized water in the final stage of cleaning to avoid ionic contamination from normal Water and maintain the cleanliness level throughout the process concentrated cleaner can be added periodically to maintain the pH level 6 to 7.9

### OUR PRODUCT ENCASING TYPE

Packaging is available in 5 and 20 Litres (HDPE White carboy Container)

### HEALTH AND SAFETY

Observe standard precautions for handling. Eye and skin protection must be provided. Avoid breathing of liquid. For detailed information refer to the relevant Health and Safety Data Sheet (MSDS) available on request.